# Real Flight Operations Guide

CanSat 2026 — Team 1079 — Manchester Satellite Development Group

XBee 3 Pro @ 9600 baud (Serial6)   |   USB Serial fallback @ 115200 baud   |   Team ID: 1079

# PART A — Commands via XBee Radio (Normal Operations)

Connect the Ground Station XBee transmitter and aim the antenna at the launch pad. All commands are ASCII lines sent at 9600 baud. The CanSat echoes the last accepted command in the CMD_ECHO telemetry field so you can confirm receipt.

## Pre-Launch Sequence — perform in order on the pad

Step 1 — Enable telemetry and reset packet counter

CMD,1079,CX,ON

Confirm: CMD_ECHO = CXON, PACKET_COUNT resets to 0, packets begin streaming at 1 Hz.

Step 2 — Set mission clock (preferred: GPS sync)

CMD,1079,ST,GPS

Or set manually: CMD,1079,ST,14:32:00   use this please
Confirm: CMD_ECHO = STGPS or ST.      this communicates back to make sure its set up properly

Step 3 — Zero the altitude (send after baro settles ~30 s)

CMD,1079,CAL

Confirm: CMD_ECHO = CAL, ALTITUDE reads 0.0 in next packet. Same here to make sure

Step 4 — Verify telemetry is healthy just read the raw telemetry coming from the cansat

Check: STATE=LAUNCH_PAD, MODE=F, ALTITUDE≈0.0, GPS_SATS≥6, MATEK=1.

Step 5 — Arm the flight software

CMD,1079,MEC,ARM,ON

Confirm: CMD_ECHO = MECARM. CanSat is now armed. Only send this when ready for launch.

Note: After ARM, the software autonomously detects launch, apogee, payload release (80% of apogee altitude), egg release (7 m AGL), and landing. No further commands are required for a nominal flight.

## Autonomous Flight — State Transitions

The software advances through these states automatically after ARM:

| Trigger | STATE in telemetry |
| --- | --- |
| Altitude > 10 m OR accel > 20 m/s² upward | ASCENT |
| 3 consecutive descending baro samples | APOGEE then DESCENT |
| Altitude ≤ 80% of apogee AGL | PAYLOAD_RELEASE |
| Altitude ≤ 7.0 m AGL | PROBE_RELEASE |
| Altitude < 0.5 m AND accel ≈ 1g (×2 samples) | LANDED |

## Emergency / Manual Override Commands (in-flight fallback only)

Only use these if the autonomous sequence fails:

Manually release payload:  CMD_ECHO: MECPAY

CMD,1079,MEC,PAYLOAD,ON

Manually release egg/probe:  CMD_ECHO: MECPRB

CMD,1079,MEC,PROBE,ON

Turn off telemetry:  CMD_ECHO: CXOFF

CMD,1079,CX,OFF

## Post-Landing

Telemetry continues streaming. STATE shows LANDED. The SD card (Flight_1079.csv) has logged the full flight regardless of XBee state. Power off the CanSat to preserve the log file.

# PART B — Commands via USB Serial (XBee Fallback)

Use this if the XBee link is unavailable. Connect a USB cable from the Teensy 4.1 to a laptop and open a serial terminal (Arduino Serial Monitor, PuTTY, etc.).

Settings: Baud rate = 115200   |   Line ending = Both NL & CR

Critical: Line ending MUST be set to 'Both NL & CR' or 'Newline'. Commands are silently ignored without a line terminator.

Commands are identical to XBee — type them exactly as shown and press Enter/Send. Every received command prints '[CMD] RX: <command>' confirming it was parsed. Telemetry also mirrors to USB Serial every second regardless of CX state.

## Full Pre-Launch Sequence via Serial

CMD,1079,CX,ON

CMD,1079,ST,GPS

CMD,1079,CAL

CMD,1079,MEC,ARM,ON

## Emergency Overrides via Serial

CMD,1079,MEC,PAYLOAD,ON

CMD,1079,MEC,PROBE,ON

CMD,1079,CX,OFF

All commands work identically to XBee. The autonomous state machine, EEPROM saves, SD logging, and XBee transmit all function normally while USB Serial is also being used.