# Simulation Mode Guide

CanSat 2026 — Team 1079 — Manchester Satellite Development Group

Simulation replaces the real barometer with Ground Station pressure values   |   All other sensors remain real

In simulation mode the CanSat ignores the real barometer and instead uses pressure values sent from the Ground Station once per second. The full flight profile (ascent, apogee, descent, landing) is replayed by streaming these values. Temperature, IMU, GPS, and battery data remain from the real hardware. MODE field reads 'S' while simulation is active.

# PART A — Simulation via XBee Radio

## Setup Sequence — perform in order

Step 1 — Enable telemetry

CMD,1079,CX,ON

CMD_ECHO = CXON

Step 2 — Arm simulation (interlock step 1 of 2)

CMD,1079,SIM,ENABLE

CMD_ECHO = SIMENABLE. MODE still shows F.

Step 3 — Activate simulation (interlock step 2 of 2)

CMD,1079,SIM,ACTIVATE

CMD_ECHO = SIMACTIVATE. Confirm MODE field now shows S.

Step 4 — Send first pressure and zero altitude

CMD,1079,SIMP,101325

CMD,1079,CAL

101325 Pa = sea level standard. ALTITUDE should read 0.0 after CAL. If your site is above sea level use actual local pressure in Pa instead.

Step 5 — Arm the state machine

CMD,1079,MEC,ARM,ON

Required if you want STATE to advance through the flight profile.

## Step 6 — Stream the Pressure Profile (1 command per second)

Send one SIMP command per second from the competition-provided CSV file. Lower pressure = higher altitude. Higher pressure = lower altitude.

| SIMP value (Pa) | Approx altitude AGL |
| --- | --- |
| 101325 | 0 m (ground) |
| 100000 | ~110 m |
| 98000 | ~285 m |
| 95000 | ~540 m |
| 90000 | ~990 m |
| 85000 | ~1460 m |
| (decrease further for higher apogee) |  |
| (then increase back toward 101325 for descent) |  |

Example sequence for a ~1460 m apogee:

CMD,1079,SIMP,101325

CMD,1079,SIMP,100000

CMD,1079,SIMP,98000

CMD,1079,SIMP,95000

CMD,1079,SIMP,90000

CMD,1079,SIMP,85000

... (apogee, now increase pressure) ...

CMD,1079,SIMP,90000

CMD,1079,SIMP,95000

CMD,1079,SIMP,98000

CMD,1079,SIMP,101325

## State Transitions During Simulation

| Altitude crosses | STATE changes to |
| --- | --- |
| Rising past 10 m AGL | ASCENT |
| 3 samples descending | APOGEE then DESCENT |
| ≤ 80% of recorded apogee | PAYLOAD_RELEASE (servo fires) |
| ≤ 7.0 m AGL | PROBE_RELEASE (servo fires) |
| < 0.5 m + accel ≈ 1g | LANDED |

## Step 7 — End Simulation

CMD,1079,SIM,DISABLE

CMD_ECHO = SIMDISABLE. MODE returns to F. Real barometer takes over immediately.

Safety interlock: SIMP values are silently ignored unless BOTH SIM,ENABLE and SIM,ACTIVATE have been sent. Sending ACTIVATE alone without ENABLE first has no effect. This prevents accidental simulation entry.

After reboot: sim_pressure_pa resets to 0 Pa, giving ALTITUDE = 44330 m. Always re-run the full sequence (ENABLE → ACTIVATE → SIMP → CAL) after any power cycle.

## CMD_ECHO Reference

| Command sent | CMD_ECHO in telemetry |
| --- | --- |
| CMD,1079,SIM,ENABLE | SIMENABLE |
| CMD,1079,SIM,ACTIVATE | SIMACTIVATE |
| CMD,1079,SIM,DISABLE | SIMDISABLE |
| CMD,1079,SIMP,101325 | SIMP101325 |
| CMD,1079,SIMP,95000 | SIMP95000 |
| CMD,1079,CAL | CAL |
| CMD,1079,CX,ON | CXON |
| CMD,1079,MEC,ARM,ON | MECARM |

# PART B — Simulation via USB Serial (XBee Fallback)

Use this if the XBee is unavailable. Connect USB to laptop, open Serial Monitor.

Settings: Baud rate = 115200   |   Line ending = Both NL & CR

Critical: Line ending MUST be set. Commands are ignored without it.

## Full Simulation Sequence via Serial

Type each command and press Enter. Watch for [CMD] RX: confirmation after each one.

CMD,1079,CX,ON

CMD,1079,SIM,ENABLE

CMD,1079,SIM,ACTIVATE

CMD,1079,SIMP,101325

CMD,1079,CAL

CMD,1079,MEC,ARM,ON

Then stream pressure values one per second (copy from competition CSV):

CMD,1079,SIMP,101325

CMD,1079,SIMP,100000

CMD,1079,SIMP,98000

CMD,1079,SIMP,95000

CMD,1079,SIMP,90000

CMD,1079,SIMP,85000

... (apogee) ...

CMD,1079,SIMP,90000

CMD,1079,SIMP,95000

CMD,1079,SIMP,101325

## End Simulation via Serial

CMD,1079,SIM,DISABLE

Telemetry is mirrored to USB Serial every second regardless of CX state — watch the ALTITUDE and STATE fields update in real time as you send each SIMP value. The [CMD] RX: line confirms each command was received and parsed correctly.

CanSat 2026 — Team 1079 — Manchester Satellite Development Group